Software Development Kit: Ephemeris Probability of Collision (Pc)

CONJUNCTION ASSESSMENT AND RISK ANALYSIS (CARA) PROGRAM

Doyle Hall, Jon Halpin

Omitron, Inc

Colorado Springs, CO

National Aeronautics and

Space Administration

Goddard Space Flight Center,

Greenbelt, MD 20771

March 2026

Preface

This document outlines the Ephemeris Pc algorithm submitted as part of the Software Development Kit (SDK). The SDK is intended to provide both industry and government customers with a code base with which to perform standard calculations inherent to the Collision Avoidance (CA) problem and as outlined in the CA Standard.

# Table of Contents

Table of Contents	iii

1.0	Introduction	4

1.1	Required Software	4

2.0	Ephemeris Pc Algorithm	5

2.1	Ephemeris Pc Calculation	5

2.2	Ephemeris Pc – Source Code Description	5

3.0	References	8

List of Tables

Table 1: Ephemeris Pc Input Parameters	5

Table 2: Ephemeris Pc Output Parameters	6

# Introduction

The CARA Software Development Kit (SDK) contains entries and artifacts for each major algorithm needed to perform the required Collision Avoidance (CA) calculations outlined in the CA Standard.  For a typical algorithm, the SDK will include a version of the algorithm, a driver program to take information from a text format CDM and execute the algorithm, producing the needed calculation or output, and a series of test cases that exercise the algorithm and produce validated results.

This document describes a specific algorithm, its associated inputs and outputs, the methodology used within the algorithm and examples of usage.

## Required Software

The following list is of software and hardware requirements for use of this SDK:

MATLAB 2019b or later

MATLAB parallel computing toolbox

MATLAB statistics and machine learning toolbox

# Ephemeris Pc Algorithm

## Ephemeris Pc Calculation

The Ephemeris Pc algorithm ingests primary and secondary orbital state and covariance ephemeris tables and calculates the statistically expected number of collisions (Nc) using the 3D-Nc method, along with bounding collision probabilities (Pcmax and Pcmin). Refer to Hall (2025) for a detailed description of the methodology and example scenarios, including low relative velocity LEO and GEO conjunctions, and a high relative velocity LEO conjunction.

The advantage of Ephemeris Pc over other Pc algorithms is that it evaluates the collision rate across the entire ephemeris overlap span, providing a unified view of the cumulative risk from multi-encounter interactions, rather than just focusing in on an individual close approach. This is particularly valuable for low relative velocity cases, where encounter segments can blend together.

The tool calculates the statistically expected collision rate  Nc(t) as a function of time throughout the ephemeris overlap span. Integrating this rate over the entire span yields the Nc.

Nc(t) is segmented into individual encounters and a segment-level Nc,k is computed for each. When the encounters are independent (not blended together), Pc,k≈Nc,k. For multi-encounter interactions, the tool provides Pcmax and Pcmin, where Pcmax assumes fully independent encounters and Pcmin assumes fully dependent encounters.

The tool also reports an accuracy indicator A that is a conservative upper limit on the uncertainty of the Nc and Pcmax estimates due to ephemeris interpolation errors or other numerical estimation inaccuracies. An accuracy indicator above 5% should raise questions about whether the ephemeris tables are adequately sampled.

## Ephemeris Pc – Source Code Description

The primary function contained within the SDK for estimating the ephemeris-based Nc and bounding collision probabilities is:

EphemerisPc.m

This function takes Consultative Committee for Space Data Systems (CCSDS) Orbit Ephemeris Message (OEM) formatted ephemerides for the primary and secondary objects as inputs. OEM ephemerides can be generated from raw ephemeris data using ‘src/CCSDSWriter.m’. There is also a Brute Force Monte Carlo (BFMC) input mode, but this is for CARA analysis team internal use only, and public users can ignore this mode since they will not have the inputs to run it. The table below will just cover the OEM mode inputs that public users will use.

The function accepts the following inputs:

Table 1: Ephemeris Pc Input Parameters

| Input Variable | Definition |
| --- | --- |
| inputPath | Path to a folder containing OEM ephemerides [Required] |
| outputPath0 | Path to an output directory where results and plots are saved [Required] |
| params | Input parameters structure with the following fields: - .HBR - Combined hard-body radius in meters [Required only for OEM mode] - .OEMPrimary - Path to primary OEM ephemeris (auto-detects Primary*.oem in inputPath if empty) [Optional] - .OEMSecondary - Path to secondary OEM ephemeris (auto-detects Secondary*.oem in inputPath if empty) [Optional] - .EphSkip - Number of ephemeris points to skip (default = 0) [Optional] - .UseEqEls - Flag for using equinoctial elements (default = true) [Optional] - .verbose - Flag for console output (default = true) [Optional] |

Outputs are stored in an EphPcOut structure. The primary outputs are:

Table 2: Ephemeris Pc Primary Output Fields in EphPcOut structure

| Output Variable | Definition |
| --- | --- |
| Nc | Statistically expected number of collisions accumulated over the risk assessment interval [Scalar] |
| Nccum | Cumulative Nc over time [1xN Vector] |
| Ncdot | Nc rate over time [1xN Vector] |
| Ncseg | Nc for each encounter segment [1xN Vector] |
| PcMax | Max collision probability accumulated over the risk assessment interval [Scalar] |
| PcMin | Min collision probability accumulated over the risk assessment interval [Scalar] |
| PcumMax | Max cumulative Pc over time [Nx1 Vector] |
| PcumMin | Min cumulative Pc over time [Nx1 Vector] |

For a full list of outputs with descriptions, reference the header of EphemerisPc.m.

EphemerisPc can be tested using ‘UnitTest/EphemerisPcUnitTest.m’.

A high relative velocity LEO conjunction scenario is saved in ‘Example’. The folder contains a slide package outlining the example, ‘EphemerisPc_Example.pptx’, a script to run the example, ‘RunExample.m’, and a folder with Inputs and Outputs for the example run, ‘HighRelVelLEO’.

# References

Hall, Doyle. (2025). Ephemeris-Based Satellite Collision Rates and Probabilities. Journal of Spacecraft and Rockets. 62. 1-18. 10.2514/1.A36227.